Techwood AV Digital Surround Receiver

Operation Manual - On-Screen Display

When your Techwood AV Digital Surround Receiver is connected to a TV monitor, you can operate the unit with the remote controller by making selections from on-screen menus.  If a menu remains on the screen for a period of time without a selection being made, the display will disappear.  Most menus disappear after 20 seconds.  Without a video signal, the screen will remain blank.

Before using the on-screen display, the ON SCREEN SELECTOR SWITCH on the rear panel of the receiver must be set to the type of video connection being used to supply video signal from your receiver to your TV monitor.

•VIDEO: Set the switch to this position when using the VIDEO MONITOR OUT jack.

• S-VIDEO: Set the switch to this position when using the S-VIDEO MONITOR OUT jack.

OSD “ON” POSITION:

1) When you turn your receiver’s power on or change its functions, a display appears on your TV monitor screen showing the current status of the system, including the type of video and audio input being monitored, the type of audio output and the status of the Tape 2 monitor. If the Tape 2 monitor is ON, the sound being heard is the sound being recorded through the Tape 2 channel.  If the Tape 2 monitor is OFF, the sound being heard is coming directly from the source of the sound.

VIDEO:VCR/VIDEO 1

AUDIO: VCR/VIDEO 1

SURROUND: STEREO

TAPE 2 MON: OFF

With this display on-screen, the type of input being monitored can be changed by pressing the appropriate key in the top section of the remote controller.  This menu will remain on the screen for only 7 seconds without input.

2) When the OSD ON button on the remote controller is pressed, a display appears which shows the current status of the input being monitored:

VIDEO INPUT: VCR/VIDEO 1

AUDIO INPUT: VCR/VIDEO 1

TAPE 2 MON: OFF

DIGITAL INPUT: ANALOG

SURROUND: STEREO

-->	GO TO MAIN MENU

MASTER VOLUME

|| || || || || || || || || || || || || ------------------------

To change these settings, press the ENTER button to bring up the MAIN MENU.  This is the only operation that can be performed on this menu, which will disappear after 7 seconds without input.

3) When MAIN MENU appears, move the arrow cursor using the up/down buttons located above and below the ENTER button to select the function you wish to change, then press ENTER.

MAIN MENU

-->	INPUT SELECTOR

DIGITAL INPUT

SURROUND MODE

SPEAKER CONFIGURATION

LANGUAGE

MAIN MENU OFF

This menu will remain on the screen for 20 seconds without input.  If it disappears before you have made a selection, press the OSD ON button then the ENTER button to bring it back.

4) Entering INPUT SELECTOR from the MAIN MENU brings up a display that allows you to select the video input that you’d like to monitor.  Move the arrow using the up/down buttons, then change the input source with the left/right buttons located on either side of the ENTER button.

INPUT SELECTOR

-->	VIDEO: VCR/VIDEO 1

AUDIO: VCR/VIDEO 1

TAPE 2 MON: OFF

GO TO MAIN MENU

To get back to the MAIN MENU move the arrow to GO TO MAIN MENU and press ENTER

5) Entering DIGITAL INPUT from the MAIN MENU brings up a menu that allows you to select the audio input you’d like to monitor.

MAIN MENU

INPUT SELECTOR

-->	DIGITAL INPUT

SURROUND MODE

SPEAKER CONFIGURATION

LANGUAGE

MAIN MENU OFF

An asterisk (*) appears opposite the input currently selected.  To change this selection, move the arrow to the input you want and press ENTER. The asterisk (*) will now appear opposite the input you have selected.

DIGITAL INPUT

INPUT: DVD/VIDEO 4

*-->	ANALOG

DIGITAL 1

DIGITAL 2

DIGITAL 3

GO TO MAIN MENU

For PHONO, TUNER, MD/TAPE 1, and VCR/VIDEO 1 inputs, ANALOG is the only selection available.

To get back to the MAIN MENU move the arrow to GO TO MAIN MENU and press ENTER.

6) Entering SURROUND MODE from the MAIN MENU brings up a menu that allows you to select the acoustical effects you’d like to use.

MAIN MENU

INPUT SELECTOR

DIGITAL INPUT

-->	SURROUND MODE

SPEAKER CONFIGURATION

LANGUAGE

MAIN MENU OFF

The choices available to you will vary according to the function and digital input you are monitoring.

SURROUND MODE

-->	SURROUND: DOLBY DIGITAL

DELAY TIME

CENTER: 5 ms

REAR: 15 ms

NIGHT -M: OFF

GO TO MAIN MENU

• When you are monitoring PHONO, TUNER, MD/TAPE 1, or VCR/VIDEO 1, use the left/right buttons to select one of these SURROUND MODE choices:

PRO LOGIC

3 STEREO

THEATER

STADIUM

HALL

DISCO

CHURCH

• When you are monitoring TV/VIDEO 2, AUX/VIDEO 3, DVD/VIDEO 4, or CD, use the left/right buttons to select one of these SURROUND MODE choices:

DTS

DOLBY DIGITAL

PRO LOGIC

3 STEREO

THEATER

STADIUM

HALL

DISCO

CHURCH

• When you are monitoring ANALOG input, use the left/right buttons to select one of these SURROUND MODE choices:

PRO LOGIC

3 STEREO

THEATER

STADIUM

HALL

DISCO

CHURCH

When you have selected DOLBY DIGITAL as your SURROUND MODE, the menu will include options for adjusting the DELAY TIME for each speaker channel.  Using the right/left buttons, the delay for the CENTER speaker can be set from 0 to 5ms and the REAR speakers can be set from 0 to 15 ms.    In the DOLBY PRO LOGIC mode, the REAR speakers can be set for 15 to 30 ms of delay.  NIGHT MODE, which compresses the audio signal to allow all sounds to be heard at a lower volume, can be changed only in the DOLBY DIGITAL mode:

To get back to the MAIN MENU move the arrow to GO TO MAIN MENU and press ENTER.

7) Entering SPEAKER CONFIGURATION from the MAIN MENU brings up a menu that allows you to select the speaker configuration you’d like to use.

MAIN MENU

INPUT SELECTOR

DIGITAL INPUT

SURROUND MODE

-->	SPEAKER CONFIGURATION

LANGUAGE

MAIN MENU OFF

Your choices will vary according to the SURROUND MODE you are using.  Use the up/down buttons to move the arrow and the right/left buttons to make your selections.

SPEAKER CONFIGURATION

-->	FRONT: LARGE

CENTER: SMALL

REAR: SMALL

SUB: ON

GO TO MAIN MENU

•In the case of DTS or DOLBY DIGITAL, your choices are:

FRONT: SMALL or LARGE

CENTER: NONE or SMALL or LARGE

REAR:  SMALL or LARGE

SUB: ON or OFF

•In the case of DOLBY PRO LOGIC, your choices are:

FRONT: SMALL or LARGE

CENTER: NONE or SMALL or LARGE

REAR:  SMALL or LARGE

SUB: ON or OFF

•In the case of 3 STEREO, your choices are:

FRONT: SMALL or LARGE

CENTER: SMALL or LARGE

REAR: NONE

SUB: ON or OFF

8) Entering LANGUAGE from the MAIN MENU brings up a menu that allows you to change the language of the on screen menus.

MAIN MENU

INPUT SELECTOR

DIGITAL INPUT

SURROUND MODE

SPEAKER CONFIGURATION

-->	LANGUAGE

MAIN MENU OFF

An asterisk (*) appears opposite the language currently selected.  To change this selection, move the arrow to the language you want and press ENTER.  The asterisk (*) will now appear opposite the language you have selected.

LANGUAGE

*-->	ENGLISH

FRENCH

GERMAN

OSD “OFF” POSITION

Test Tone: In DOLBY DIGITAL or DOLBY PRO LOGIC SURROUND MODES, pressing the TEST TONE button  on the remote controller brings up a display which allows you to adjust the relative volume of front, rear and center speaker channels.

TEST TONE

CENTER: 0dB

FL	C	FR	REAR

R: 0dB

L: 0dB

RL	SW	RR	SUB: 0dB

When the display comes up, each channel will begin to blink in turn.  When a channel is blinking, you can raise or lower its volume using the up/down buttons.  To turn the display off, press ENTER or TEST TONE.

System Status: To display the current status of your delay times and volume settings, press any buttons on the remote controller related to SURROUND, including SURROUND ON/OFF, SURROUND MODE, LEVEL SELECT, SUB W ON/OFF, SUB W LEVEL, CENTER/REAR DELAY or the up/down buttons.  This display will remain on screen for 7 seconds.

SURROUND: DOLBY DIGITAL

DELAY TIME

CENTER: 5ms

REAR: 15 ms

NIGHT-M: OFF

LEVEL TRIM

CENTER: -5dB

SUB: 0dB

REAR L: +5 dB

REAR R: 0dB

Once this display is up, pressing the CENTER or REAR DELAY TIME, or the CENTER or REAR LEVEL buttons will enable you to change the function of the button pressed using the right/left buttons. Pressing SUB W ON/OFF or SUB W LEVEL will allow you to change the sub-woofer setting by pressing the same button a second time.  Without input, the display will disappear in 7 seconds.

Sleep: Pressing the SLEEP button  on the remote controller brings up a display that allows you to select the amount of time from 90 to 0 minutes after which the system will shut itself off.  Once the display is up, each time you press the SLEEP button reduces the time in ten minute increments.  At zero, pressing the SLEEP button again  starts the sequence over at 90 minutes.

SLEEP: _ _

Without input, the display will disappear in 7 seconds.

Volume: Pressing the MASTER VOLUME button on the remote controller brings up a display that allows you to adjust the volume by pressing on the upper or lower half of the MASTER VOLUME button.

MASTER VOLUME

|| || || || || || || || || || || -------------------------

Without input, the display will disappear in 7 seconds.